# Signals

- What is the purpose of expressing an AC signal as an RMS value?

To give an AC value a relatable comparison to DC voltages and currents.

The AC RMS value is equivalent to DC value that would give the same heating effect (i.e. through a resistance).

- What factors affect the reliable transmission of a signal through a communications channel?

Noise and interference are the major factors.  Other factors include attenuation in transmission lines, phasing effects, jitter in timed digital transmission lines.

- How many 12kHz channels can be multiplexed into a 120kHz frequency band?

10

- How many compressed channels can be multiplexed into a 120kHz channel (discuss …)

Open ended discussion!

It depends on the level of compression used.

However, transmission channels are subject to a Shannon maximum effect which is not really to do with compression itself, but is the limit of data/information possible in a channel with given transmission parameters (modulation level, etc) and the noise level (signal/noise ratio) in that channel

- Explain the occurrence and effect of jitter in a communications channel.

inaccuracies in electronic timing circuits in transmission or receiver devices

electromagnetic interference

crosstalk with carriers of other signals

tolerable jitter - depends on the affected application

- What is meant by “symbols” in data transmission?

A symbol is the single element of data to be transmitted as specified for a specific data link.

For digital transmission a symbol may consist of one or more bits as determined by the modulation format

Examples

In a BPSK. system, each symbol represents 1 bit

QPSK Quadrature phase shift keying, symbol represents 2 bits

etc

64QAM: 64 distinct symbols; i.e. 2^6 = 64; i.e. 6 bits/symbol

- What is intersymbol interference in (a) a pulsed digital channel and (b) a radio transmission channel where data is sent as symbols represented on a carrier?

The distortion of the signal when one symbol interferes with subsequent symbols

Caused by data spread (e.g. phase distortions causing the signal to extend) in transmission media

Also caused by reflections in radio transmissions, specifically causing multipath effects, resulting in the signal (symbol) and its reflections to arrive at the receiver at different times (the direct signal being the first to arrive).  Subsequent reception of reflections of the same signal will add to the received signal (constructivey or destructively) to cause potentially ambiguous symbols requiring decoding.

A typical method of avoiding this is to allow a guard time between symbols. This will slightly reduce the data carrying bandwidth of the channel (i.e. if the channel was perfect), but will  aleviate the ISI problem which would otherwise reduce the effective transmission efficacy of the channel either due to the requirement for retransmissions, or in data being irretrievable lost.  (However, lost data may be further safe-guarded (to a certain extent) by the other techniques, e.g. FEC).

- What is an eye diagram and what performance characteristics does it diagnose in a communications channel?

An eye diagram can demonstrate the performance of a data channel.

Repeated binary signals can build up a “history” of reception which, when “overlaid” in time can demonstrate the effect of:

Noise – demonstrated by the amount of overlap of signals reducing the apparent height between binary 1 and binary 0

Jitter – demonstrated by the rising and falling edges of the data thickening the vertical edges of the pulses.

- Express the following in binary and hexadecimal:

Dec		Bin		Hex

- 7 	            0111		  7

- 23	          10111		17

- 67	      1000011		43

- 123	      1111011		78

- 1007	1111101111		3EF

Express the following in base 10 (standard decimal) numbering

Bin		Hex		Dec

- 1001		 9		  9

- 1100		 C		12

- 10001		11		17

- 10101		15		21

- 11000		18		24

Hex		Dec		Bin

- #9A		154		10011010

- #BC		188		10111100

- #29		  41		    101001

- #45		  69		  1000101

- #4C                 76		  1001100