## Broadcast Technology UG1 Tutorial

# Tutorial: Revision – Compression

Q1

What is the percentage of data saving in the 4:2:2 chroma subsampling process?

(assume the original RGB and derived YCrCb planes are all the same bit depth (for example each is 8 bits deep = 24b colour)

SOLUTION

A third of the sub-pixels are removed, i.e. 33.3% saving

Q2

Considering MPEG-2 Transport Streams (TS) and Program Streams (PS):

- Where are these each used

- Discuss the differences between the two formats and their relative advantages for the role the provide.

SOLUTION

Program Streams are used in secure environments (e.g. production facilities)

Transport Streams are used in transmission over networks (cable, air) where losses are more likely to occur.

Program Streams contain their data in PS “packs” – these are fairly long packets of data.  There is no specific limit on their length, but generally they are used for media data of around 0.5 – 0.7 seconds

Transport Stream data is limited to packets of 188 bytes (plus another 16B of error correction).  The consideration is that it is better that data losses are ept to smaller amounts at a time. It is easier to track and correct data errors in the smaller packets.

Q3

Explain the concept of a psychoacoustic model and its application to audio compression

SOLUTION

The model is a generalised representation of the way the human auditory system perceives noise

The diagram shows the (A) the threshold of hearing; (C) a signal in the audio; (D) adjacent tones/noise, and (B) the envelope modifying the threshold of hearing for that adjacent tone in the presence of signal (C).

D has to be sufficiently strong to clear the modified envelope in order to be perceived

The nearer to the signal C, the adjacent D has to be louder to be detected

Critical bands – the narrow range of frequencies the ear detects these occurrences.

The signal is subject to quantisation noise which spans the full frequency range; consequently the processes breaks the signal into critical bands

e.g. an adjacent signal sufficiently far from C (in freq) would not have to be so strong to be perceived.

C is the masking signal

D is the masked signal

The codec uses these proportions of C and D to determine the bit rate used to code D; either it will decide D is completely masked, or masked such that only a small bit rate is needed to represent the signal.

Q4

What aspects of human visual physiology are addressed in video compression?

SOLUTION

Compression applied in chroma subsampling assumes that the human eye registers a much higher resolution of an image (and the world in general) in monochrome; i.e. the light intensity detected by the eye. This is in comparison to the human eye’s perception of the colour of the image. This is because the eye has a large number of ROD sensors (120 million, they perceive the light intensity of the image) compared to only about 7 million CONE sensors which detect the colour/wavelength of the image.

Consequently we can compress the colour data of an image substantially ore than we can compress the image intensity (i.e. the high resolution monochromatic view we have).

So we take the initial three layers of the image: the red-green-blue as captured in the sensor; and immediately convert it to a monochrome layer (“Luma”, or “Y”) and two other colour layers which are actually colour difference signals. i.e. The Cr layer is the Red colour difference: it is actually the value of Red at a any given point minus the value of the luma (i.e. Cr=R-Y). The Cb is the blue colour difference layer; it is the Blue valu of the image at any given point minus the Luma (Cb = B-Y)

This means we can now treat the luma and the chroma as two different entities.  We can leave the luma intact whilst aggressively decimating the two colour layers and so reducing the data of the image.

Following this there is an even more intense compression activity – the DCT-based compression: This will be dealt with in year 2 of the course.