# Basic Transmission

## 1

What is the period of

- a 1kHz signal?

- a 200 kHz signal

- a 5MHz Signal

## 2

What is the relation between wavelength, frequency and velocity of propagation

## 3

What is self inductance, and how does it affect the transmission of a signal along a conductor?

## 4

What other electrical characteristics affect the transmission of a signal in a long length of cable?

## 5

What is total internal reflection? How does it affect the propagation of light in a fibre?

## 6

If a pulse of white light source is shone into a fibre, how is the pulse affected as the pulse travels along the fibre?

## 7

What are the differences between propagation of a sound wave and a radio wave?

What is the relationship between the two fields in EM radiation?

## 8

What is the purpose of twisting pairs of conductors together in twisted pair cable?

## 9

What special precautions are to be taken when installing and deploying fibre optic cable?

## 10

How does destructive and constructive interference affect radio wave reception?

Extended Exercise

## 11

This demonstration explores how self-inductance affects signal transmission in a simple RL conductor model, treating it as a first-order low-pass filter with transfer function H(s) = R/(Ls + R). The code presents three perspectives on this effect: the step response showing how quickly the system responds to an input change, the square-wave response illustrating how fast edges become rounded and smoothed, and the Bode magnitude and phase plots revealing frequency-dependent attenuation above the cutoff frequency. This foundational model helps students understand that inductance opposes rapid current changes, and that even simple conductors behave as filters that attenuate high-frequency components while introducing phase lag.

Run this demo in Matlab and observe the output.

clc; clear; close all;

% =========================================================

% Demo 1: Self-inductance effect on signal transmission

% Model: simple RL conductor

%

% Theory:

%   v_in(t) = R*i(t) + L*di/dt

%   v_out(t) = R*i(t)

%

% Transfer function:

%   H(s) = Vout/Vin = R / (L*s + R)

%

% Interpretation:

%   The conductor behaves like a first-order low-pass system.

%   Increasing L makes the response slower and smooths fast edges.

% =========================================================

%% Parameters

R = 50;            % resistance in ohms

L = 200e-6;        % inductance in henries

% Transfer function

num = R;

den = [L R];

sys = tf(num, den);

%% Time constant

tau = L / R;

fprintf('Time constant tau = L/R = %.6e s\n', tau);

%% Part A: Step response

figure('Name','Demo 1A - Step Response');

step(sys);

grid on;

title('Step Response of an RL Conductor Model');

xlabel('Time (s)');

ylabel('Output Voltage (V)');

%% Part B: Square-wave transmission

f = 2000;                          % square-wave frequency in Hz

t = 0:1e-6:3e-3;                   % time vector

vin = square(2*pi*f*t);            % input square wave

vout = lsim(sys, vin, t);          % output after RL conductor

figure('Name','Demo 1B - Square Wave Distortion');

plot(t*1e3, vin, '--', 'LineWidth', 1.2); hold on;

plot(t*1e3, vout, 'LineWidth', 1.5);

grid on;

title('Effect of Self-Inductance on a Square Wave');

xlabel('Time (ms)');

ylabel('Voltage (V)');

ylim([-1.5 1.5]);   % or use [-2 2] if you want more margin

legend('Input square wave','Output after RL conductor','Location','best');

%% Part C: Frequency response

figure('Name','Demo 1C - Frequency Response');

bode(sys);

grid on;

title('Frequency Response of RL Conductor Model');

%% notes printed in command window

fprintf('\n Interpretation:\n');

fprintf('1. Self-inductance produces a voltage L*di/dt that opposes current change.\n');

fprintf('2. Fast transitions are resisted, so edges become rounded.\n');

fprintf('3. This introduces delay, high-frequency attenuation, and phase shift.\n');

## 12

This comprehensive simulation analyses pulse propagation through an RLCG transmission line by solving a lumped ladder network of 20 sections. The demo generates multiple plots that show how an input pulse evolves over time at different positions (input, mid-line, and output), and also displays voltage profiles along the entire cable at several snapshots in time. Through these visualizations, students observe key transmission line effects: attenuation from series resistance, delay from energy storage in inductance and capacitance, and dispersion or spreading of the pulse wavefront, providing physical insight into why real communication channels and power distribution systems exhibit frequency-dependent behaviour and signal degradation.

Run this demo in Matlab and observe the output.

clc; clear; close all;

% =========================================================

% Demo 2: RLCG transmission line using a lumped ladder model

%

% One small section of cable is modelled by:

%   - series R and L

%   - shunt G and C

%

% Repeating many sections approximates a long transmission line.

%

% State variables:

%   i_k(t)   = current through section k series branch

%   v_k(t)   = node voltage at section k

%

% We solve the differential equations numerically using ode45.

% =========================================================

%% Cable parameters per section

N  = 20;              % number of sections

R  = 2;               % series resistance per section (ohms)

L  = 20e-6;           % series inductance per section (H)

C  = 200e-12;         % shunt capacitance per section (F)

G  = 1e-6;            % shunt conductance per section (S)

RL = 100;             % load resistance at end (ohms)

%% Input signal: pulse

Vpulse = 1;           % pulse amplitude (V)

pulse_width = 20e-6;  % pulse width (s)

vin = @(t) Vpulse * (t >= 5e-6 && t <= (5e-6 + pulse_width));

%% State vector x:

% x = [i1 i2 ... iN  v1 v2 ... vN]^T

x0 = zeros(2*N,1);

%% Simulation time

tspan = [0 200e-6];

%% Solve ODE

opts = odeset('RelTol',1e-6,'AbsTol',1e-8);

[t, x] = ode45(@(t,x) rlcg_line_ode(t, x, N, R, L, C, G, RL, vin), tspan, x0, opts);

%% Extract currents and voltages

I = x(:,1:N);

V = x(:,N+1:end);

v_in  = arrayfun(vin, t);

v_mid = V(:, round(N/2));

v_out = V(:, N);

%% Plot input, mid-point, output

figure('Name','Demo 2A - Pulse Propagation');

plot(t*1e6, v_in,  '--', 'LineWidth', 1.2); hold on;

plot(t*1e6, v_mid, 'LineWidth', 1.5);

plot(t*1e6, v_out, 'LineWidth', 1.5);

grid on;

title('Pulse Propagation Along an RLCG Transmission Line');

xlabel('Time (\mus)');

ylabel('Voltage (V)');

legend('Input pulse','Mid-line voltage','Output voltage','Location','best');

%% Plot voltage snapshots along the line at selected times

sample_times = [20e-6, 50e-6, 100e-6, 150e-6];

figure('Name','Demo 2B - Voltage Along the Cable');

hold on;

for k = 1:length(sample_times)

[~, idx] = min(abs(t - sample_times(k)));

plot(1:N, V(idx,:), 'LineWidth', 1.5, ...

'DisplayName', sprintf('t = %.0f us', sample_times(k)*1e6));

end

grid on;

title('Voltage Distribution Along the Transmission Line');

xlabel('Section Number');

ylabel('Node Voltage (V)');

legend('Location','best');

%% Print teaching notes

fprintf('RLCG transmission line simulation complete.\n');

fprintf('\nTeaching interpretation:\n');

fprintf('1. R causes attenuation along the cable.\n');

fprintf('2. L resists fast current changes.\n');

fprintf('3. C stores charge between conductors, causing delay and pulse spreading.\n');

fprintf('4. G models leakage through imperfect insulation.\n');

fprintf('5. Repeated RLCG sections produce distortion and spreading of the pulse.\n');

%% =========================================================

% Local function: ODE model of the RLCG ladder

% =========================================================

function dxdt = rlcg_line_ode(t, x, N, R, L, C, G, RL, vin)

i = x(1:N);          % series currents

v = x(N+1:2*N);      % node voltages

dxdt = zeros(2*N,1);

% Input source voltage

Vs = vin(t);

% --- Series branches: di/dt equations ---

for k = 1:N

if k == 1

v_left = Vs;

else

v_left = v(k-1);

end

v_right = v(k);

% L di/dt = v_left - v_right - R*i

dxdt(k) = (v_left - v_right - R*i(k)) / L;

end

% --- Shunt nodes: dv/dt equations ---

for k = 1:N

i_in = i(k);

if k < N

i_out = i(k+1);

i_load = 0;

else

i_out = 0;

i_load = v(k) / RL;   % load current at final node

end

% C dv/dt = i_in - i_out - G*v - i_load

dxdt(N+k) = (i_in - i_out - G*v(k) - i_load) / C;

end

end

## 12

This demonstration animates voltage pulse propagation along a transmission line using a lumped ladder circuit model consisting of cascaded RLCG sections. The code models a series of transmission line segments, each with resistance R, inductance L, series capacitance C, and shunt conductance G, solving the coupled differential equations numerically to show how an input voltage pulse travels and distorts along the cable length. The animation displays the voltage distribution at discrete time intervals, providing intuitive visualization of wave propagation effects and how the pulse shape changes due to the combined influence of series dissipation and shunt leakage.

Run this demo in Matlab and observe the output.

clc; clear; close all;

% =========================================================

% RLCG Transmission Line Animation

% Visualise pulse propagation along a cable

% =========================================================

%% Parameters

N  = 40;              % number of sections (more = smoother)

R  = 2;

L  = 50e-6;

C  = 500e-12;

G  = 1e-6;

RL = 100;             % load

%% Input pulse

Vpulse = 1;

pulse_width = 20e-6;

vin = @(t) Vpulse * (t >= 5e-6 && t <= (5e-6 + pulse_width));

%% Initial state

x0 = zeros(2*N,1);

%% Time

tspan = [0 150e-6];

opts = odeset('RelTol',1e-6,'AbsTol',1e-8);

[t, x] = ode45(@(t,x) rlcg_line_ode(t, x, N, R, L, C, G, RL, vin), tspan, x0, opts);

%% Extract voltage

V = x(:,N+1:end);

%% Animation

figure('Name','Transmission Line Animation');

for k = 1:10:length(t)   % skip frames for speed

plot(1:N, V(k,:), 'LineWidth', 2);

ylim([-0.2 1.2]);

grid on;

title(sprintf('Pulse Propagation at t = %.1f µs', t(k)*1e6));

xlabel('Position along cable (section)');

ylabel('Voltage (V)');

drawnow;

end

%% =========================================================

% ODE function

% =========================================================

function dxdt = rlcg_line_ode(t, x, N, R, L, C, G, RL, vin)

i = x(1:N);

v = x(N+1:2*N);

dxdt = zeros(2*N,1);

Vs = vin(t);

% Series branches

for k = 1:N

if k == 1

v_left = Vs;

else

v_left = v(k-1);

end

v_right = v(k);

dxdt(k) = (v_left - v_right - R*i(k)) / L;

end

% Shunt nodes

for k = 1:N

i_in = i(k);

if k < N

i_out = i(k+1);

i_load = 0;

else

i_out = 0;

i_load = v(k) / RL;

end

dxdt(N+k) = (i_in - i_out - G*v(k) - i_load) / C;

end

end